Technical Bulletin

Subject: VWC Remanufactured 2.0l Air-Cooled Engine Group: 10

Number: 10-90 T01

Models(s): Type II/Vanagon 1975 - 83 MY Date: May 31, 1990

Engines, P/N 071 100 031AX, 039 100 031 B/C/DX, have been modified to improve heat transfer an increase durability. These changes effect Engine codes: GD, GE, CV.

The following improvements have been introduced as of remanufactured Engine number 89000.

- To eliminate seizing of pistons, increased piston/cylinder clearance from 0.03mm  0.008mm to 0.045mm  0.005mm.

- Eliminate aluminum seal 021 101 341A between cylinder and cylinder head.

- Eliminate paper gasket 021 101 341A between foot of cylinder and crankcase.

- Install 1.6mm aluminum shim 071 101 341, with adhesive D 000 400 at foot of cylinder to compensate for removed paper gasket and aluminum seal. Illustration 1.

- To improve heat transfer, oil groves on both sides of the connecting rod to allow oil splash to cool bottom of the piston. Illustration 2